Quiz no 6

| A/A | Question | Type |
| --- | --- | --- |
| 1 | For frequency modulation (FM) the ratio of the maximum frequency deviation from the carrier to the maxim frequency of the modulating signal is called: - the modulation index. - upper sideband. - lower sideband. - the frequency bandwidth. Correct Answer: A | Multiple Choice |
| 2 | Each of the bits of digital information is transmitted during a fixed time period called: - baud. - tempo. - clock. - matrix. Correct Answer: A | Multiple Choice |
| 3 | On-off keying (OOK) can be considered a special case of pulse amplitude modulation (PAM) in which are two RF signal amplitudes one for a “one” and the other for a “zero.” Correct Answer: True | True/False |
| 4 | In FSK, one frequency in a baud transmits a “one” and another frequency transmits a “zero.” Correct answer: True | True/False |
| 5 | In phase modulation–binary phase shift keying (BPSK) the signal has one phase during a one and changes phase during a zero by: - 180 degrees. - 90 degrees. - 60 degrees. - 45 degrees. Correct Answer: A | Multiple Choice |
| 6 | In quadrature phase shift keying (QPSK) the receiver detects a particular phase in the received signal and outputs: - two bits of digital data. - four bits of digital data. - one bits of digital data. - six bits of digital data. Correct Answer: A | Multiple Choice |
| 7 | Low probability of intercept (LPI) communication signals are also called: - spread spectrum signals. - special signals. - encrypted signals. - communication signals. Correct Answer: A | Multiple Choice |
| 8 | Modulations which provide multiple bits per baud are considered high efficiency modulations because they allow more digital data to be transmitted in any given RF bandwidth. Correct answer: True | True/False |
| 9 | A receiver designed for direct sequence spread spectrum signal reception has a despreading demodulator which applies the same pseudo-random signal that was applied by the transmitter. Correct answer: True | True/False |
| 10 | The GPS global navigation system transmits its signals with: A. direct sequence spectrum spreading. B. chirped signal. C. frequency hopping signal. D. pseudo-random codes. Correct Answer: A | Multiple Choice |